SECTION 32 31 17

ORNAMENTAL WELDED WIRE FENCING

*****  Ametco® Manufacturing Corporation manufactures several types of ornamental metal fencing.  This guide specification section can be used to specify ornamental welded wire galvanized steel security fencing.  Other types of Ametco® fencing can be specified with SECTION 32 31 20 - ORNAMENTAL STEEL FENCING, SECTION 32 31 21 - ALUMINUM LOUVER FENCING, and SECTION 32 31 22 - STAINLESS STEEL LOUVER FENCING.  Ametco® also manufactures a railing system which can be specified in SECTION 05 73 10 - OPEN GRILLE STEEL RAILING SYSTEM.

Ametco® manufactures four types of welded wire security fencing which meet various security levels.  Security is classified by Basic, Medium, and High categories.  The types of welded wire fencing contained in this guide section and their recommended security categories are:

Fiesta Design 		High Security

Guardian Design	Medium and High Security

Park Design		      Basic and Medium Security

Amopanel Design     Basic and Medium Security

The specifier will need to edit this product specification for a specific project to reflect the options and applications being used.  The guide section has been written so that most editing can be accomplished by deleting unnecessary requirements.  Options are indicated by  [   ].  Notes to assist the specifier in selecting options and editing the specification guide are printed in bold and indicated with  *****.  For final editing, all brackets and notes will need to be deleted from the guide.

******************************************************************************

PART 1 - GENERAL

1.1	SUMMARY

A.	Section includes:  Welded wire mesh, modular fencing panels including tubular steel posts and gates.

B.	Related sections:

*****  List other specification sections dealing with work directly related to this section such as the following.  *****

1.	Section 03 30 00 - Cast-in-Place Concrete:  Concrete footings for support of fence posts.

1.2	REFERENCES

****  List by number and full title reference standards referred to in remainder of specification section.  Delete non-applicable references.  *****

A.	American Society for Testing and Materials (ASTM) Publications:

1.	ASTM A36 - Structural Steel.

2.	ASTM A123 - Zinc (Hot-Dip Galvanized) Coatings on Iron and Steel Products.

3.	ASTM A500 - Cold-Formed Welded and Seamless Carbon Steel Structural Tubing in Rounds and Shapes.

4.	ASTM A526 - Sheet Steel, G-90 Zinc-Coated (Galvanized) by the Hot-Dip Process.

5.	ASTM A792 - Steel Sheet, Aluminum-Zinc Alloy-Coated (Galavlume) by the Hot Dip Process.

6.	ASTM B117 - Standard Practice for Operating Salt Spray (Fog) Apparatus.

7.	ASTM D822 - Tests on Paint and Related Coatings Using Filtered Open-Flame Carbon-Arc Exposure Apparatus.

8.	ASTM D1794 - Resistance of Organic Coatings to the Effects of Rapid Deformation (Impact).

9.	ASTM D3363 - Test Method for Film Hardness by Pencil Test.

1.3	SUBMITTALS

A.	Provide in accordance with Section 01 33 00 - Submittal Procedures:

1.	Product data for components and accessories.

2.	Shop drawings showing layout, dimensions, spacing of components, and  anchorage and installation details.

3.	Sample:  [8 by 10 inches] [203 by 254 mm] minimum size sample of fence panel illustrating design, fabrication workmanship, and selected color coating.

4.	Copy of warranty specified in Paragraph 1.4 for review by Architect.

1.4	WARRANTY

A.	Provide in accordance with Section 01 77 00 - Closeout Procedures:

1.	Factory finish:  20 years warranty against cracking, peeling, and blistering under normal use.

PART 2 - PRODUCTS

2.1	ACCEPTABLE MANUFACTURERS

A.	Ametco® Manufacturing Corporation, 4326 Hamann Parkway, P.O. Box 1210, Willoughby, Ohio 44096; 800-362-1360.

B.	Requests to use equivalent products of other manufacturers shall be submitted in accordance with Section 01 63 00 - Product Substitution Procedures.

2.2	MATERIALS

A.	Wire: Galvanized steel wire.

B.	Steel bar stock:  ASTM A36.

C.	Steel tubing:  ASTM A500, Grade B.

D.	Steel sheet:  ASTM A526 galvanized or ASTM A792 Galvalume.

E.	Grout:  Non-shrink type, pre-mixed compound consisting of non-metallic aggregate, cement, and water reducing and plasticizing additives.

2.3	FENCE SYSTEM

*****  Ametco® Manufacturing Corporation provides 6 types of welded wire security fence systems providing different levels of security.  Refer to Ametco® product literature for descriptions, illustrations, and recommended applications.

Use the following to specify Fiesta Design recommended for high security applications.  *****

A.	Type:  Ornamental steel fencing system consisting of welded wire mesh modular fence panels, tubular steel posts, and U-shaped clamping strips and designed for high security applications; Fiesta Design as manufactured by Ametco® Manufacturing Corporation.

B.	Fence panels:  Fabricated from galvanized wire welded to form open mesh.

1.	Vertical wires:  [1/4 inch] [6 mm] diameter spaced at [2 inches.]  [51 mm.]

2.	Horizontal wires:  Two [5/16 inch] [8 mm] diameter wires encasing vertical wires and spaced at [8 inches.]  [203 mm.]

3.	Pickets:  Form by extending vertical wires [1 inch] [25 mm] above top horizontal wire.

*****  Fiesta Design panels are provided in 3 standard heights.  Panel width is 98-13/16 inches (2510 mm).  Panels can be cut to accommodate non-standard width.  *****

4.	Size:  [[98-13/16 inches] [2518 mm] [_____] wide by [[48] [72] [96] inches] [[1219] [1829] [2438] mm] high.

5.	Panel fastener:  Full height, U-shaped clamping strip with slots to accommodate horizontal wires and fastened to posts with safety screws anchoring panels to post face.

******************************************************************************

Use the following to specify Guardian Design recommended for both medium and high security applications.  *****

C.	Type:  Ornamental steel fencing system consisting of welded wire mesh modular fence panels, tubular steel posts, and strap type panel fasteners and designed for medium and high security applications; Fiesta Design as manufactured by Ametco® Manufacturing Corporation.

D.	Fence panels:  Fabricated from galvanized wire welded to form open mesh.

1.	Vertical wires:  [1/4 inch] [6 mm] diameter spaced at [2 inches.]  [51 mm.]

2.	Horizontal wires:  Two [5/16 inch] [8 mm] diameter wires encasing vertical wires and spaced at [8 inches.]  [203 mm.]

3.	Pickets:  Form by extending vertical wires [1 inch] [25 mm] above top horizontal wire.

*****  Guardian Design panels are provided in 3 standard heights.  Panel width is 98-13/16 inches (2510 mm).  Panels can be cut to accommodate non-standard width.  *****

4.	Size:  [[98-13/16 inches] [2518 mm] [_____] wide by [[48] [72] [96] inches] [[1219] [1829] [2438] mm] high.

5.	Panel fastener:  K-shaped strap fastener, synthetic retainer, and safety screw to anchor panels to face of post.

******************************************************************************

Use the following to specify Park Design recommended for both basic and medium security applications.  Park Design is similar to Guardian Design except for lighter wire diameter.  *****

E.	Type:  Ornamental steel fencing system consisting of welded wire mesh modular fence panels, tubular steel posts, and strap type panel fasteners and designed for basic and medium security applications; Fiesta Design as manufactured by Ametco® Manufacturing Corporation.

F.	Fence panels:  Fabricated from galvanized wire welded to form open mesh.

1.	Vertical wires:  [3/16 inch] [5 mm] diameter spaced at [2 inches.]  [51 mm.]

2.	Horizontal wires:  Two [1/4 inch] [6 mm] diameter wires encasing vertical wires and spaced at [8 inches.]  [203 mm.]

3.	Pickets:  Form by extending vertical wires [1 inch] [25 mm] above top horizontal wire.

*****  Guardian Design panels are provided in 3 standard heights.  Panel width is 98-13/16 inches (2510 mm).  Panels can be cut to accommodate non-standard width.  *****

4.	Size:  [[98-13/16 inches] [2518 mm] [_____] wide by [[48] [72] [96] inches] [[1219] [1829] [2438] mm] high.

5.	Panel fastener:  K-shaped strap fastener, synthetic retainer, and safety screw to anchor panels to face of post.

******************************************************************************

*****  Use the following to specify Amopanel Design.  *****

G.	Type:  Ornamental steel fencing system consisting of welded wire mesh modular fence panels with formed reinforcing ribs, tubular steel posts, and strap type panel fasteners and designed for basic security applications; Amopanel Design as manufactured by Ametco® Manufacturing Corporation.

H.	Fence panels:  Fabricated from [3/16 inch] [5 mm] diameter galvanized wire welded to form open mesh.

1.	Vertical wire spacing:  [2 inches.]  [51 mm.]

2.	Horizontal wire spacing:  [8 inches.]  [203 mm.]

3.	Reinforcing ribs:  V shaped, horizontal ribs formed by bends in vertical wires and 3 welded horizontal wires.  Locate at top and bottom of fence panel and at intermediate points as required by panel height.

4.	Pickets:  Form by extending vertical wires [1 inch] [25 mm] above top horizontal wire.

*****  Amopanel Design panels are provided in 4 standard heights.  Panel width is  98-13/16 inches (2510 mm).  Panels can be cut to accommodate non-standard width.  *****

5.	Size:  [[98-13/16 inches] [2518 mm] [_____] wide by [[48] [55] [78] [95] inches] [[1219] [1397] [1981] [2413] mm] high.

6.	Panel fastener:  K-shaped strap fastener, synthetic retainer, and safety screw to anchor panels to face of post.

******************************************************************************

2.4	POSTS

*****  Posts for Guardian Design and Amopanel Design are profiled on four sides with vertical, V-shaped, recessed channels to receive wire edge of fence panel.  Edit the following to reflect fence panel being specified.  *****

A.	Type: Steel tubes fabricated from ASTM A526 galvanized steel or ASTM A792 galvalume steel as selected by manufacturer for type and size of fencing.  [Provide post faced with vertical, V-shaped, recessed channels to receive wire edge of fence panel.]

B.	Size:

1.	Fence posts:  [2 by 2 inches.] [51 by 51 mm.]

2.	Gate posts:  [4 by 4 inches.] [102 by 102 mm.]

*****  50 and 62 inches (1270 and 1575 mm) high posts are 14 gage (2.1 mm).  78 inches (1981 mm) posts are 12 gage (2.8 mm) and 96 inches (2438 mm) posts are 11 gage (3 mm).  *****

C.	Wall thickness:  [[14] [12] [11] gage.]  [[2.1] [2.8] [3] mm.

*****  Posts are provided in 6 standard lengths to accommodate different fence heights.  *****

D.	Length:  [[59] [71] [[86] [98] [106] [118] inches.] [[1499] [1803] [2184] [2489] [2692] [2997] mm.]

E.	Equip posts with plastic top caps.

F.	Factory drill posts to receive fence panel attachment fasteners.

2.5	GATES

*****  Single and double hinged swinging gates in 4 standard heights are provided by  Ametco® Manufacturing Corporation.  Edit the following paragraphs to reflect required gates.  *****

A.	Type:  Hinged swinging [single] [double] gate.

B.	Construction:  Welded frame fabricated from [2 by 2 inches] [51 by 51 mm] steel tubing with welded wire panel to match fencing.

C.	Nominal size:  [[48] [96] [144] inches] [[1219] [2438] [3658] mm] wide by [[50] [62] [78] [96] inches]  [[1270] [1575] [1981] [2438] mm] high.

D.	Hardware:  Equip gates with manufacturer's standard hardware required for functional operation.

1.	Hinges:  Size and type as determined by manufacturer.  Provide 2 hinges for each leaf up to [6 feet] [1829 mm] height and 1 additional hinge for each additional [24 inches] [610 mm] in height or fraction thereof.

2.	Latch:  [3/4 inch] [19 mm] slide bolt to accommodate padlock.

3.	For double gates provide padlockable, center cane bolt assembly and strike.

2.6	FACTORY FINISH

A.	Steel fence panels and posts shall be hot-dip galvanized to 1.25 ounces per square foot minimum zinc coating in accordance with ASTM A123 or provided with galvalume coating complying with ASTM A792.  Standard size components shall receive polyester powder coating.  Large gate panels shall be coated with 2-part polyurethane coating.

B.	Polyester powder coating:  Electrostatically applied colored polyester powder coating heat cured to chemically bond finish to metal substrate.

1.	Minimum hardness measured in accordance with ASTM D3363:  2H.

2.	Direct impact resistance tested in accordance with ASTM D2794.  Withstand 160 inch-pounds.

3.	Salt spray resistance tested in accordance with ASTM B117:  No undercutting, rusting, or blistering after 500 hours in 5 percent salt spray at 95 degrees F and 95 percent relative humidity and after 1000 hours less than [3/16 inch] [5 mm] undercutting.

4.	Weatherability tested in accordance with ASTM D822:  No film failure and 88 percent gloss retention after 1 year exposure in South Florida with test panels tilted at 45 degrees.

C.	Polyurethane coating:  1.0 mil dry film thickness of coating of steel test panel cured 30 minutes at 180 degree F and aged 14 days shall resist the following test conditions without failure:

1.	5 percent salt spray for 500 hours.

2.	100 percent relative humidity for 1000 hours.

3.	Water immersion for 100 hours.

4.	20 double rubs with cloth saturated with either lacquer thinner, acetone, MEK, gasoline, xylene.

5.	Exposure to lubricating oils, hydraulic fluids, and cutting oils.

6.	16 cycles of 24 hours at 100 percent humidity, 24 hours at 10 degrees F, and 24 hours at 77 degrees F.

7.	Hardness:  H to 2H.

8.	Flexibility:  [1/8 inch] [3 mm] conical mandrel.

*****  Ametco® Manufacturing Company provides 15 standard colors.  Custom colors are available for minimum size orders.  Contact Ametco® Manufacturing Company for information on custom colors.  *****

D.	Color:  [[Red Baron] [Black Velvet] [Blue Streak] [Bronze Mat] [Cal Gray]  [Charlie Brown] [Cream]  [Light Blue] [Light Ivory] [Moss Green] [Pale Green] [Safety Orange] [Safety Yellow] [Silver with weather resistant clear coating] [White] as manufactured by Ametco® Manufacturing Company.] [Selected by Architect from manufacturer's standard range.] [Custom color as selected by Architect.]

PART 3 - EXECUTION

3.1	PREPARATION

A.	Prior to fabrication, field verify required dimensions.

*****  Size of concrete fence footings will depend on fence height, post spacing, and other project conditions.  Footing dimensions may be indicated on Drawings or in this section but not both locations in order to eliminate potential conflicts.  Edit the following paragraphs to reflect specific project conditions.  *****

B.	Cast concrete footings in accordance with Section 03 30 00 - Cast-in-Place Concrete as detailed on Drawings and approved shop drawings.

1.	Minimum footing diameter:

a.	Terminal and gate posts:  [12 inches.] [305 mm.]

b.	Intermediate line posts:  [10 inches.] [254 mm.]

2.	Allow [8 inches] [203 mm] [_____] minimum embedment of posts.

3.	Allow [6 inches] [152 mm] [_____] minimum concrete beneath post bottom.

C.	[Provide setting holes for embedment of fence posts.]  [Core drill existing concrete footings for embedment of fence posts.]  Hole shall be [2 inches] [51 mm] minimum greater than post width.

3.2	INSTALLATION

A.	Install fencing in accordance with manufacturer's installation instructions and approved shop drawings.

B.	Install fence posts plumb and level [by setting post in hole [cast] [drilled] in concrete and grouting solid.]  [by embedding post directly in concrete footing.]  Temporarily brace fence posts with 2 by 4 wood supports until [concrete] [grout] is set.

C.	Do not installed bent, bowed, or otherwise damaged panels.  Remove damaged components from site and replace.

*****  Panels can be attached to posts prior to or after setting. posts or after setting posts, panels can be attached with adjustable brackets.  *****

D.	Install fence panels to posts [prior to setting posts in footings.] [after posts have been set in footings and [concrete] [grout] is cured.

*****  Edit the following paragraph to reflect fastener required for specific type of fence panel.  *****

1.	Secure wire fence panels with [K-shaped strap fastener] [U-shaped clamping strip] [hook screws] U-shaped clamping device] [fastener type as recommended by manufacturer.]

2.	Space fasteners and secure to posts at spacing indicated on approved shop drawings.

3.	Attach channel shaped connectors over joints between abutting horizontal bars.

E.	Install gates and adjust hardware for smooth operation.  [Provide concrete center foundation depth and drop rod retainers at center of double gate openings.]

F.	Attach plastic caps to steel posts.

G.	Touch-up damaged finish with paint supplied by manufacturer and matching original coating.

END OF SECTION